Azure migrate Activities

Login into portal.azure.com

Go to Azure Migrate  Server migration Replicating Machines   Select the server and click on Migrate

Say yes to the shutdown the VMs in Vcenter for full replication.

Monitor the migration status.

Login into Venter and uncheck the network , this VM is already  power off by Migration process

Once the server is up under virtual machines, navigate use Boot diagnostics  to see the server is up now

Set the diagnostics storage account.

Update the Tags for the server as mentioned in the inventory sheet.

Login into the server using the ip address for the first time

Enable the diagnostic settings as required.

Enter the credentials and login into the system.

Auto Shutdown  the VM, if required it is non-prod VMs.

Uninstall the Vmware tools in the environment , as mentioned in the document.

Set the Pagefile to system managed  size in d drive which is created after the migration.

Remove the Page file settings in C drive and other drives and reboot the server.

Add Vms in Logs analytics workspace for MRCGlobalOMS workspace.

Make sure that connect is grayed.

Confirm the Azure backup as required in  Migration sheet.